# STANDARD OPERATING PROCEDURE

Temperature Control Testing for ASHES

Purpose: Controlled temperature variation for sensor accuracy testing and fire detection auto-activation verification

# SECTION 1: OVERVIEW

## 1.1 PURPOSE

This SOP describes procedures for safely and accurately changing the ambient temperature around DFRobot SHT31 sensors to:

- Verify sensor accuracy across temperature range

- Test fire detection auto-activation thresholds

- Validate rate-of-change detection algorithms

## 1.2 SCOPE

Applies to:

- ASH1 sensor node testing

- ASH2 sensor node testing

- ASH0 fire detection logic validation

- Integration testing of the multi-node system

## 1.3 SAFETY PRECAUTIONS

WARNING: High temperature testing involves heat sources and electronic components. Follow all safety procedures carefully.

- Monitor tests continuously - never leave unattended

- Never exceed 85 °C - sensor maximum operating temperature 125 °C

- Keep heat sources away from battery and other electronics

- Use insulated tools when handling hot equipment

- Have a fire extinguisher readily available

- Work in a well-ventilated area

- Avoid skin contact with heated surfaces

- Allow cooling period before handling

# SECTION 2: EQUIPMENT AND MATERIALS

## 2.1 REQUIRED EQUIPMENT

Testing Equipment:

- Reference thermometer

- Reference hygrometer

- Timer or stopwatch

- Multimeter (for battery voltage verification)

- Laptop with Arduino IDE (or similar) and Serial Monitor

- USB cable for serial connection

Temperature Control Methods (choose based on test type):

- Method A: Heat Gun (for rapid temperature increase)

- Method B: Environmental Chamber (if available)

Support Equipment:

- Thermally insulated enclosure

- Data logging sheet or Excel file

- Camera or phone for documentation

## 2.2 SENSOR NODE HARDWARE

- ASH1 or ASH2 node (Heltec LoRa v3 with SHT31 sensor)

- Charged battery or USB power supply

- ASH0 control node (for auto-activation testing)

- Backup power cables

## 2.3 ENVIRONMENTAL CONDITIONS

Baseline conditions before test:

- Room temperature: 20-25°C

- Relative humidity: 30-60%

- No direct sunlight on sensors

- Minimal air movement

- Stable ambient conditions for 5 minutes prior

# SECTION 3: PRE-TEST PROCEDURES

## 3.1 SYSTEM PREPARATION

- Power on sensor node and verify initialization:

Expected Serial output:

- "SHT31 sensor initialized successfully."

- No RADIOLIB_OR_HALT errors

- Normal LoRa transmission at 15-second mark

- Record baseline readings (let stabilize for 2 minutes):

Time: ____5:20________

Temperature: ____22.71__°C (sensor)      vs ___21.9___°C      (reference)

Humidity:        ___42.04___%RH (sensor) vs __n/a____%RH (reference)

Battery:            ___n/a___V

- Verify reference instruments are calibrated and stable (do the reported readings seem reasonable?)

yes

- Position reference thermometer within 10cm of SHT31 sensor

- If testing auto-activation:

- Verify ASH0 is powered and connected to serial monitor and the internet.

- Confirm ASH0 receiving ASH1 transmissions (check Serial)

- Note current counter value

- Verify solenoid is in closed state

- Clear any existing fire detection array data if needed

# SECTION 4: TEMPERATURE CONTROL METHODS

## 4.1 METHOD A: HEAT GUN (Rapid Temperature Rise)

Use for: Rate-of-change fire detection testing (3-5°C rise in <2 minutes)

Procedure:

- Set heat gun to LOW setting (avoid direct contact)

- Position heat gun 15-30cm from sensor node

- Apply heat in 15-second intervals

- Move heat gun in slow circular motion

- Monitor temperature every 15 seconds

- Stop when target temperature reached

CAUTION: Heat gun can exceed 100°C quickly - use careful distance control and exposure time

## 4.2 METHOD B: ENVIRONMENTAL CHAMBER

Use for: Testing across full temperature range

If environmental chamber available:

- Place sensor node and reference instrument inside chamber

- Set target temperature and humidity

- Allow 1 minutes for stabilization

- Record readings at steady state

- Increment temperature in 10°C steps

- Test range: ~20°C to +70°C (within triggering limits)

# SECTION 5: DATA RECORDING AND ANALYSIS

## 5.1 DATA COLLECTION

For each test, record:

- Date and time

- Test type and objective

- Node ID (ASH0, ASH1, ASH2)

- Message counter value at start

- Temperature profile (all readings)

- Humidity profile (all readings)

- Battery voltage

- Trigger events and timing

- Any anomalies or unexpected behavior

## 5.2 SERIAL OUTPUT CAPTURE

Save complete Serial Monitor output for:

- Sensor readings every 15 seconds

- Array contents display

- Fire detection messages

- Auto-trigger activation

- Any error messages

# SECTION 6: POST-TEST PROCEDURES

## 6.1 COOL-DOWN AND RESET

- Turn off heat source immediately after test completion

- Allow sensor node to cool naturally to room temperature

- Expected cool-down time: 5-15 minutes depending on max temperature reached

- Verify sensor returns to baseline

- Reset message counter if needed using Serial command 'r'

- Reset internal timer if needed using Serial command 't'

## 6.2 HARDWARE INSPECTION

After high-temperature testing (>50°C):

- Inspect sensor for physical damage

- Check for discoloration or melting

- Test I2C communication

- Verify LoRa transmission resumes normally

- Check battery voltage for degradation

- Inspect PCB for heat damage

- Verify SHT31 sensor still responds

## 6.3 FUNCTIONAL VERIFICATION

After cooling, verify:

- Sensor initializes successfully on next boot

- Temperature readings return to normal range

- Humidity readings return to normal range

- LoRa transmission continues at 15-second intervals

- No persistent error messages

## 6.4 DATA ARCHIVAL

- Save all recorded data to Excel test plan file

- Save Serial Monitor logs with timestamp

- Document any issues or anomalies

- Photograph sensor node condition

- Update test summary

- File data in project folder with test date

# SECTION 7: TROUBLESHOOTING

## 7.1 SENSOR NOT RESPONDING TO TEMPERATURE CHANGE

Possible causes:

- Insufficient heat transfer (sensor not in heated zone)

- Thermal lag (need to wait longer for stabilization)

- Sensor failure or I2C communication error

Solutions:

- Move heat source closer (maintain safe distance)

- Increase heating time

- Verify sensor initialization in Serial output

- Check connections (sensor on GPIO 45, 46)

## 7.2 HUMIDITY NOT DROPPING AS EXPECTED

Possible causes:

- Ambient humidity too high

- Insufficient temperature rise

- Heat source adding moisture (steam)

Solutions:

- Increase temperature change magnitude

- Test in drier environment

- Use desiccant to lower baseline humidity

- Accept that humidity trigger may be difficult to test without chamber

## 7.3 AUTO-TRIGGER NOT ACTIVATING

Possible causes:

- Temperature below threshold (not actually ≥65°C)

- Humidity not low enough

- Array not full (for rate-of-change tests)

- Both conditions not met simultaneously

- autoTriggerActive remains true from previous test

- Firmware logic errors

Solutions:

- Verify actual temperature with reference thermometer

- Check array contents in Serial output

- Ensure LocalDataCount =  4 for rate-of-change

- Verify both temperature AND humidity conditions met

- Reset system and clear arrays before retrying

- Check ASH0 is receiving LoRa transmissions

- Check ASH0 is not stuck in reconnect loop

- Review firmware

## 7.4 TEMPERATURE OVERSHOOT

Possible causes:

- Heat source too close

- Heat applied too rapidly

- Insufficient monitoring

Solutions:

- Increase distance from heat source

- Use lower heat setting or airflow

- Apply heat in shorter intervals

- Monitor more frequently

- Use gentle heating method (hair dryer vs heat gun)

## 7.5 SENSOR DAMAGED BY EXCESSIVE HEAT

### Symptoms:

- Sensor reads constant value (stuck)

- I2C communication failure

- Sensor initialization fails

- Erratic readings

### Prevention:

- Never exceed 85°C maximum operating temperature

- Monitor continuously during heating

- Use gradual temperature increase

- Keep heat away from other components

- Have reference thermometer close to sensor

### Recovery:

- If the sensor is damaged, replacement is likely required

- Review HandlingInstructions.pdf pg. 5 -Reconditioning Procedure